# POV Bike Wheel LED Display — Hardware Build Guide

Target: 26" cruiser test bike / future AMYET fat tire e-bike

# Power Architecture

IMPORTANT: There are two completely separate circuits. Never combine them.

## Riding Circuit (wheel spinning):

LiPo battery → MT3608 boost converter (5.07V) → XIAO ESP32-S3 + LED strip

## Charging Circuit (parked at camp):

USB-C wall charger → TP4056 charger board → LiPo battery

The TP4056 is NEVER in the riding circuit. It is a standalone charger only. At camp: unplug LiPo from MT3608, plug into TP4056 to charge overnight.

# Key Hardware Decisions

| Component | Choice | Notes |
| --- | --- | --- |
| MCU | XIAO ESP32-S3 | WiFi, MicroPython, tiny 21×17mm |
| LEDs | SK9822 144/m Black PCB IP30 | Dual clock = SPI compatible |
| Power | LiPo 3.7V 2000mAh 654065 | JST PH 2.0mm connector |
| Boost converter | MT3608 | Adjusted to 5.07V output |
| Charger | TP4056 Type-C | Charging only — not in ride circuit |
| Hall sensor | A3144 | GPIO4 (D3), PULL_UP, IRQ_FALLING |
| Arm material | 1/8" × 1/2" aluminum flat bar | Home Depot ~$6 for 3ft |
| Connectors | Pre-made JST PH 2.0mm cables | Do NOT use crimp tool — buy pre-made |

# Wiring Reference

## SK9822 Wire Colors

| Wire | Signal | XIAO Pin |
| --- | --- | --- |
| Red | 5V Power | 5V pin |
| Black | GND | GND pin |
| Yellow | CI (Clock) | D8 / GPIO7 |
| Green | DI (Data) | D10 / GPIO9 |

## Hall Sensor (A3144) — flat face toward you, legs down

| Leg | Label | Connect to |
| --- | --- | --- |
| Left | VCC | XIAO 3.3V |
| Middle | GND | XIAO GND |
| Right | OUT | XIAO D3 (GPIO4) |

## Full Wiring Chain

LiPo (+)   → MT3608 VIN+

LiPo (-)   → MT3608 VIN-

MT3608 VOUT+ → XIAO 5V

MT3608 VOUT- → XIAO GND

MT3608 VOUT+ → LED strip 5V (Red wire)

MT3608 VOUT- → LED strip GND (Black wire)

XIAO D10   → LED strip DI (Green wire)

XIAO D8    → LED strip CI (Yellow wire)

XIAO 3.3V  → Hall sensor VCC (Left leg)

XIAO GND   → Hall sensor GND (Middle leg)

XIAO D3    → Hall sensor OUT (Right leg)

# Phase 1 — Build the LED Arm

1. Cut aluminum flat bar (1/8" × 1/2") to measured spoke length

2. File the mounting face lightly (3-4 passes) to roughen surface for adhesion

3. Wipe with isopropyl alcohol, let dry 30 seconds

4. Cut SK9822 LED strip to match — count LEDs, cut only at marked scissor points

5. Peel backing, press LED strip firmly onto filed face, hold 60 seconds

6. Leave data-in end (DI/CI/5V/GND pads) exposed at hub end of arm

# Phase 2 — Wire the Hub Assembly

Do NOT use breadboard for final build — solder all connections directly.

1. Set MT3608 to 5V FIRST — connect LiPo only, measure VOUT, turn trim pot (many turns) until reading 5.0-5.1V. 5.07V is perfect.

2. Solder all connections per wiring chain above

3. Hall sensor: solder VCC and GND legs permanently, leave OUT leg on temporary wire until GPIO pin confirmed

4. Bench test: power on from LiPo, confirm XIAO boots, LEDs light, hall sensor counts rotations

# Phase 3 — Mount to Wheel

1. Position aluminum bar radially spanning two adjacent spokes (hub to rim)

2. Zip tie at hub end and rim end — between LEDs, not over them

3. Mount hall sensor at hub END of arm, flat face pointing toward fork leg

4. Stack 2-3 neodymium magnets, mount to fork leg at exact same height as hall sensor

5. Spin wheel by hand — confirm sensor passes within 3-5mm of magnet each rotation

6. Mount XIAO + MT3608 + LiPo near hub with velcro + zip ties

# Troubleshooting Reference

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| LEDs don't light | Wrong end of strip connected | Check arrow — connect to DI/CI input end |
| LEDs don't light | XIAO inserted backwards | Flip XIAO around |
| Only some LEDs light / wrong colors | Loose DI or CI wire | Wiggle wires, resolder |
| Hall sensor always reads 1 | Breadboard connection issue | Solder directly — no breadboard |
| Hall sensor always reads 0 | Magnet too close or wrong pole | Move magnet away, or flip it over |
| MT3608 voltage not changing | Need many pot turns | Turn slowly, many full rotations required |
| XIAO resets while riding | Power brownout | Check MT3608 voltage under load |

# Lessons Learned

- Breadboards are unreliable for this build — solder everything directly

- Pre-made JST PH 2.0mm cables only — do NOT buy crimping tools or loose connector kits

- TP4056 is charge-only — never in the riding power circuit

- Check LED strip direction — must connect to input (DI/CI) end, not output (DO/CO)

- XIAO orientation matters — easy to insert backwards on a breadboard

- MT3608 trim pot needs many turns — not a small adjustment

- Carbon fiber bar is impractical — use 1/8" × 1/2" aluminum flat bar instead

- 30 LEDs/m strip is wrong density — must order BTF-5V-144L-B (144/m)