POV Bike Wheel

Firmware & Upload Guide

MicroPython · VS Code · XIAO ESP32-S3 · Two-arm build

This guide covers the two-arm build: two LED strips per arm, four strips total, daisy-chained on a single SPI bus. No extra code complexity — the daisy-chain means all strips see the same data automatically.

# Part 1 — Set Up VS Code

Write all code on your MacBook in VS Code, upload to the XIAO over USB-C.

Figure 1 — VS Code to XIAO via USB-C

## Step 1.1 — Install the MicroPico extension

- VS Code → Extensions (⌘+Shift+X) → search MicroPico by paulober → Install

- A connection status indicator appears in the bottom status bar

## Step 1.2 — Flash MicroPython onto the XIAO

- Download Generic ESP32-S3 MicroPython firmware (.bin) from micropython.org/download

- Connect XIAO via USB-C

- Bootloader mode: hold BOOT, tap RESET, release BOOT

- VS Code Command Palette (⌘+Shift+P) → MicroPico: Flash MicroPython → select the .bin file

- Reset board — MicroPico status bar shows Connected

💡 Use a data-capable USB-C cable, not a charge-only cable. If status shows Disconnected, check the cable first.

# Part 2 — File Structure

Same four files as a single-arm build. The two-arm change is entirely in the wiring — the daisy-chain handles the second strip at the hardware level, so the firmware doesn't know or care how many strips are physically connected.

Figure 2 — four files, same structure as single-arm build

- apa102.py — SPI driver, unchanged from single-arm

- hall_sync.py — rotation interrupt, unchanged

- frames.py — your artwork, unchanged

- main.py — main loop, one small addition: NUM_LEDS doubles to 78 (39 per arm × 2 arms)

💡 Because the strips are daisy-chained, writing 78 pixels in one SPI transaction lights the first 39 LEDs on strip 1 and the next 39 on strip 2 automatically. No extra SPI calls needed.

# Part 3 — Write apa102.py (LED Driver)

Identical to single-arm. The SPI driver sends a single stream of pixel data — the daisy-chain distributes it across both strips transparently.

Figure 3 — SK9822 SPI data format, same for 2 or 4 strips

## The code — unchanged from single-arm

from machine import SPI, Pin

class APA102:

def __init__(self, spi_id=1, sck_pin=7, mosi_pin=9,

num_leds=78, brightness=20):  # 78 = 39 x 2 arms

self.num_leds = num_leds

self.brightness = brightness & 0x1F

self.spi = SPI(spi_id, baudrate=8_000_000,

polarity=0, phase=0,

sck=Pin(sck_pin), mosi=Pin(mosi_pin))

def show(self, pixels):

data = bytearray(4)           # start frame

for r, g, b in pixels:

data += bytes([0xE0 | self.brightness, b, g, r])

data += bytes([0xFF] * ((self.num_leds // 16) + 1))

self.spi.write(data)

def off(self):

self.show([(0, 0, 0)] * self.num_leds)

## Test it

from apa102 import APA102

strip = APA102(num_leds=78)

strip.show([(255, 0, 0)] * 78)    # all 4 strips should go red

strip.off()

💡 If only the first two strips light up, the daisy-chain connection between strip 2 and strip 3 has a wiring issue. Check the data-out → data-in solder joint between them.

# Part 4 — Write hall_sync.py (Rotation Detector)

Completely unchanged from single-arm. One sensor, one magnet, one interrupt per rotation regardless of arm count.

Figure 4 — rotation timing, same for any number of arms

from machine import Pin

import time

class HallSync:

def __init__(self, pin_num=3):

self.pin = Pin(pin_num, Pin.IN, Pin.PULL_UP)

self.last_time = time.ticks_us()

self.period_us = 0

self.rotation_count = 0

self.pin.irq(trigger=Pin.IRQ_FALLING,

handler=self._on_trigger)

def _on_trigger(self, pin):

now = time.ticks_us()

diff = time.ticks_diff(now, self.last_time)

if diff > 5000:

self.period_us = diff

self.last_time = now

self.rotation_count += 1

def get_period(self): return self.period_us

def is_spinning(self):

return time.ticks_diff(time.ticks_us(), self.last_time) < 1_000_000

## Test it

from hall_sync import HallSync

h = HallSync(pin_num=3)

# wave magnet past sensor — check count increases

print(h.rotation_count, h.period_us)

# Part 5 — Write frames.py (Image Data)

One key change for two-arm: each column now has 78 pixels (39 per arm) instead of 39. The first 39 pixels go to arm 1, the next 39 to arm 2 via daisy-chain.

Figure 5 — image conversion pipeline, same process

## Test pattern — updated for 78 LEDs

NUM_COLUMNS = 60

NUM_LEDS = 78       # 39 per arm x 2 arms

LEDS_PER_ARM = 39

def make_test_frame():

frame = []

for col in range(NUM_COLUMNS):

color = (255, 0, 0) if col % 2 == 0 else (0, 0, 255)

# same pixel data repeated for both arms via daisy-chain

frame.append([color] * NUM_LEDS)

return frame

FRAMES = [make_test_frame()]

## Image converter — updated for 78 LEDs

Run this on your MacBook. Source image should be 60 x 78 pixels (60 columns x 78 LEDs). The top 39 rows go to arm 1, bottom 39 rows to arm 2.

from PIL import Image

NUM_COLUMNS = 60

NUM_LEDS = 78

def image_to_frame(path):

img = Image.open(path).convert('RGB')

img = img.resize((NUM_COLUMNS, NUM_LEDS))

frame = []

for col in range(NUM_COLUMNS):

column_pixels = []

for led in range(NUM_LEDS):

r, g, b = img.getpixel((col, led))

column_pixels.append((r, g, b))

frame.append(column_pixels)

return frame

def write_frames_file(paths, output='frames.py'):

with open(output, 'w') as f:

f.write(f'NUM_COLUMNS = {NUM_COLUMNS}\n')

f.write(f'NUM_LEDS = {NUM_LEDS}\n\n')

f.write('FRAMES = [\n')

for p in paths:

f.write('    ' + repr(image_to_frame(p)) + ',\n')

f.write(']\n')

write_frames_file(['my_logo.png'])

💡 Design your source image at 60 x 78 pixels. The top half (rows 0-38) maps to arm 1, bottom half (rows 39-77) maps to arm 2. For a symmetrical design both halves show the same image — for a creative build they can show different content.

# Part 6 — Write main.py (Main Loop)

Almost identical to single-arm. The only change is NUM_LEDS=78. Everything else — timing, frame advance, kill switch button, hall sync — is the same.

Figure 6 — main loop execution flow, unchanged

## The code

import time

from machine import Pin

from apa102 import APA102

from hall_sync import HallSync

from frames import FRAMES, NUM_COLUMNS, NUM_LEDS

strip = APA102(num_leds=NUM_LEDS, brightness=20)  # 78 LEDs

hall = HallSync(pin_num=3)

button = Pin(4, Pin.IN, Pin.PULL_UP)

current_frame = 0

rot_at_last_frame = 0

ROTATIONS_PER_FRAME = 5

leds_on = True

last_btn = 0

def check_button():

global leds_on, last_btn

if button.value() == 0:

now = time.ticks_ms()

if time.ticks_diff(now, last_btn) > 300:

leds_on = not leds_on

if not leds_on: strip.off()

last_btn = now

while True:

check_button()

if not leds_on or not hall.is_spinning():

strip.off()

time.sleep_ms(50)

continue

period = hall.get_period()

if period == 0:

time.sleep_ms(10)

continue

if hall.rotation_count - rot_at_last_frame >= ROTATIONS_PER_FRAME:

current_frame = (current_frame + 1) % len(FRAMES)

rot_at_last_frame = hall.rotation_count

column_dur = period // NUM_COLUMNS

frame = FRAMES[current_frame]

for col in range(NUM_COLUMNS):

t0 = time.ticks_us()

strip.show(frame[col])

elapsed = time.ticks_diff(time.ticks_us(), t0)

remaining = column_dur - elapsed

if remaining > 0:

time.sleep_us(remaining)

# Part 7 — Upload All Files

## Upload order

- Connect XIAO via USB-C — confirm MicroPico shows Connected

- VS Code Explorer → MicroPico Device panel

- Right-click each file → Upload to MicroPico

- Upload order: apa102.py → hall_sync.py → frames.py → main.py

- Press RESET on the XIAO — main.py runs automatically

💡 To update artwork only: regenerate frames.py on your laptop using the converter script, upload just frames.py, reset. The other three files stay on the board unchanged.

## What to expect on first boot with two arms

- Both arms light up simultaneously — the daisy-chain distributes data to all four strips in one SPI transaction

- Image appears twice per rotation (once per arm) — looks brighter and more defined than single-arm

- If one arm shows the image and the other shows nothing — check the data-out to data-in connection between strip 2 and strip 3

- If both arms show image but they look offset from each other — arm 2 is not mounted exactly 180° opposite arm 1, adjust its position

# Quick Troubleshooting

- Only first arm lights up → daisy-chain break between strip 2 data-out and strip 3 data-in

- Arms show image at different positions → arms not 180° apart, or hall sensor triggers inconsistently

- LEDs don't light at all → wrong SCK/MOSI pin numbers in APA102.__init__

- hall.rotation_count never increases → wrong GPIO pin or sensor wiring issue

- Image blurry → reduce NUM_COLUMNS from 60 to 40

- Board resets while spinning → power brownout, check MT3608 output under load

- ModuleNotFoundError → file not uploaded to board, re-upload via MicroPico file browser

- SyntaxError on boot → typo in one of the .py files, check REPL output for line number